THE KINETICS AND SPECIFC ENERGY CONSUMPTION OF MELON SEEDS DRYING

1Nwajinka C.O and 2Oluka S.I. and 1Okpala, C.D

1Nnamdi Azikiwe University, Awka, Anambra State, Nigeria.

2Enigu State University of Technology, Enugu State, Nigeria.

Abstract: Drying models were developed to predict the drying characteristics of melon seeds at various air temperature, velocity, and relative humidity. To achieve this, a laboratory model dryer was developed and instrumentally equipped to determine the effect of drying conditions on the measured parameters such as drying rate constants and effective diffusion coefficient. The drying rate decreased with increases in drying time and air temperature. The moisture diffusivity was found to increase with air temperature and velocity, which follows the trend for most agricultural and food materials. The moisture diffusivity of serewa variety was observed to vary from 4.194×10–13 m2/s at the air velocity of 1.3 m/s and air temperature of 29°C to maximum of 2.108× 10–10 m2/s at the air velocity of 1.3 m/s and air temperature of 55°C. The specific energy consumption for drying the seeds were 1.209 x 1010, 5.04 x 1010, 8.085 x 1010 and 1.752 x 1010,J/kgwater, for temperatures of 40, 50, 55, and 60 oC respectively. Thermal efficiency of the drying system increased from 12% up to 51.96% in the temperature range of 29 oC to 55 oC after which it began to fall, while the pick-up efficiency was  26%.

Keywords: Dryer, models, drying kinetics, rate constants, moisture diffusivity